# METHOD OF STATEMENT

Date: 23rd Feb 2024

Description of work

To temporary relocate 1 unit machine and put aside, using lorry crane to assist dismantle machine part within factory premise

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/02/24 – 01/03/24

Time of Operation

10.00am

Location of Operation

#### 70 Joo Koon Circle

Nakano

Equipment Use

1 x lorry loader

1 x boom lift

Hydraulic jack

Woodblock

Rollers

Metal plate (for epoxy floor protection)

1 x forklift

Obstacles

Ensure no obstacles around operation area

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# To jack up machine to 4” high

# Insert rollers under machine bed

# Shift out from existing location

# Transport to temporary location within factory premise

# Transport in lorry loader into factory premise

# To assist engineer dismantle machine parts for repair purpose

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175