# METHOD OF STATEMENT

Date: 21st Jan 2021

Description of work

To relocate 1 unit Mixer from existing location to another location within production room, jack up 60 cm for engineer to install stand

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 5 Sunview Rd

Equipment Use

Crowbar

Woodblock

1 x 5 ton pallet truck

Obstacles

Ensure no obstacles around operation area and route to new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Lift up mixer to 3” high

# Insert pallet truck under mixer base

# Jack up pallet truck to ensure mixer leg do not hit the ground

# Shift out mixer from existing location

# Transfer to new location within production room

# Position mixer at the new designated location

# Jack up mixer for engineer to install stand

# Lower down mixer and let it sit onto stand

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175