# METHOD OF STATEMENT

Date: 23rd Dec 2024

Description of work

To relocate 5 units machine from existing location to new location within factory premise

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

TBC

Time of Operation

TBC

Location of Operation

#### 14 Gul Dr

Premier Hytemp

Equipment Use

1 x lorry loader

1 x 3ton forklift

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

1 set of drilling equipment

1 set hand drill

Metal plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out during heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# Deploy lorry loader at designated unloading area

# To hoist down forklift from lorry chassis to the ground

# Unload necessary equipment for the job

# Jack up machine to 4” high

# Insert rollers under machine bed

# Shift out machine from existing location

# Transport machine to back of building within factory premise

# Use of forklift to assist in pulling/pushing machine

# Drill hole for low plug

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Repeat process till all machine have been relocate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175