# METHOD OF STATEMENT

Date: 26th Aug 2025

Description of work

To relocate 4 units machine from existing location to new location within factory premise

Machine Spec

1 unit Youji YV1000 ATC-2

1 unit Hwacheon L2-2500

1 unit Mazak VRX-i500

1 unit Mazak Vortex I630 V6-3

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

30/09/25 – relocate 1 unit Mazak VRX-i500

TBA – relocate Hwacheon L2-2500, Youji YV1000, Mazak Vortex I630 V6-3

Time of Operation

10:00 am

Location of Operation

#### 2A Seletar Aerospace Link

Avimac Pte Ltd

Equipment Use

1 x lorry loader

Hydraulic jack

Woodblock

Electric roller/rollers

1 set hand drill

Canvas/metal plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Job Scope

# Jack up machine to 4” high

# Insert rollers under machine bed

# Shift out machine from existing location

# Transport machine to new location within factory premise

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Drill hole to insert low plug and bolt down machine using existing metal plate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175