# METHOD OF STATEMENT

Date: 03rd July 2024

Description of work

To relocate 2 unit machine within factory premise

Machine Spec

1 unit Summit Lathe / Wt: 2, 500 kg

1 unit Microweily TY2660 / Wt: 2, 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/07/24

Time of Operation

10:00am

Location of Operation

#### 23 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x lorry loader

Crowbar

Woodblock

Rollers/pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Deploy lorry loader at designated unloading area

# To unload necessary equipment for the move

# Lift up machine to 3” high

# Insert rollers/pallet truck under machine bed

# Shift out machine from existing

# Transport to new location within production area

# Unload machine from rollers/pallet truck

# Insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175