# METHOD OF STATEMENT

Date: 13th May 2024

Description of work

To relocate 1 unit machine within factory premise

Machine Spec

Dim: 350 x 300 x 350cm/ Wt: 14ton

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

1 unit Wuxi VTL Machine (14A)

Time of Operation

10:00am

Date of Operation

15/05/24

Location of Operation

#### 23 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x lorry loader

Hydraulic/hand jack

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Deploy lorry loader at designated unloading area

# To unload necessary equipment for the move

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing location to temporary location

# Contractor to drill bolt for the above machine

# To shift back machine to exact location

# Position at the designated location

# Jack up one side of machine

# Remove rollers and insert woodblock under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear and

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175