# METHOD OF STATEMENT

Date: 07th Aug 2025

Description of work

To relocate 5 units machine from existing location to new location within factory premise

Machine Spec

2 units Mazak HCN6800 / Wt: 21, 500 kg each

1 unit Hwacheon Vesta 200 / Wt: 15, 000 kg

1 unit Youji 1000 ATC / Wt: 19, 000 kg

1 unit Eumach VMC1600 / Wt: 15, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

20-21/08/25 – relocate 2 unit Mazak HCN6800

26-27/08/25 – relocate Hwacheon Vesta, Youji 1000 ATC, Eumach VMC1600

Time of Operation

10:00 am

Location of Operation

#### 2A Changi North St 2

ETLA Limited

Equipment Use

1 x lorry loader

Hydraulic jack

Woodblock

Electric roller/rollers

1 set hand drill

Canvas/metal plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Job Scope

# Jack up machine to 4” high

# Insert rollers under machine bed

# Shift out machine from existing location

# Transport machine to new location within factory premise

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Drill hole to insert low plug and bolt down machine

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175