# METHOD OF STATEMENT

Date: 24th Apr 2024

Description of work

To relocate 2 unit machine from existing location to another location within factory premise

Machine Spec

2 unit Power Master

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

25/04/24

Time of Operation

09:45 am

Location of Operation

#### 149 Gul Circle

FMC Technologies Singapore Pte Ltd

Equipment Use

1 x lorry loader

Hydraulic/hand jack

Woodblock

Rollers/electrical roller

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Deploy lorry loader at designated unloading area

# Unload necessary equipment for the job

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing location

# Transport machine to new location within factory premise

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175