# METHOD OF STATEMENT

Date: 18th June 2024

Description of work

To relocate 2 unit machine within production area

Machine Spec

Mori Seiki SL-403C/2000

Wt: 14, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

18/06/24

Time of Operation

09:45 am

Location of Operation

#### 16E Tuas Ave 1 #01-61

Hunting Energy Services Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Motor operate rollers/manual rollers

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing location

# Transport to new location within production area

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175