# METHOD OF STATEMENT

Date: 28th Mar 2023

Description of work

To relocate 2 unit machine from existing location to another location within factory premise

Machine Spec

1 unit Proma S-40-15L-C2 CG

Dim: 700 x 200 x 170 cm / Est Wt: 7-9 ton

1 unit Proma 2 CG

Dim: 1200 x 270 x 220 cm / Est Wt: 18-20 ton

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

03-04/04/23

Time of Operation

09:45 am

Location of Operation

#### 21 Loyang Crescent

Safran Landing Systems Services Singapore Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers

Hand grinder

1 x forklift

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing location

# Transport to new location within factory premise

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175