# METHOD OF STATEMENT

Date: 22nd May 2023

Description of work

To relocate 2 unit machine from existing location to another location within factory premise

Machine Spec

1 unit Ravenburg Conventional Lathe

1 unit Summit Lathe

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

Est: 06/06/23

Time of Operation

09:45 am

Location of Operation

#### 21 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing location

# Transport to new location within factory premise

# Position machine at the new designated location

# Jack up machine, insert woodblock and remove rollers

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175