# METHOD OF STATEMENT

Date: 27th May 2025

Description of work

To relocate 3 unit machine and accessories from Bay 2 to Bay 3 within production area

Machine Spec

1 unit Mazak Power Master

1 unit Mazak MTV-655/80N

1 unit Mazak Integrex E650 H/II

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

05-06/06/25 – relocate Mazak Power Master & Mazak MTV-655/80N

11-12/06/25 – relocate Mazak Integrex E650

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte Ltd

Equipment Use

Hydraulic jack

Woodblock

Motor operated rollers/manual rollers

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe)

Job Scope

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing Bay 3

# Transport to new location at Bay 2 within production area

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175