# METHOD OF STATEMENT

Date: 22nd Sep 2022

Description of work

To relocate machineries from existing location to another location within factory premise

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/09/22 (Phase 1)

1 unit Amada 358 Turret Punch

1 unit Time Saver

14-16/10/22

(Phase 2)

1 unit Aida 250ton Press

1 unit Chin Fong 300ton Press

2 unit Chin Fong 250ton Press

1 unit 150ton Single Crank Press

3 unit 110ton Press

1unit Amada 150ton Double Crank Press

Phase 3

1 unit Komatsu 150ton Single Crank Press

1 unit Aida 110ton Press

2 unit Chin Fong 110ton Press

1 unit Shing Tai 110ton Press

3 unit OCP-80N Press

1 unit 45ton Press

Time of Operation

09:45 am

Location of Operation

#### 36 Joo Koon Circle

Trend Technologies (S) Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers

Canvas/metal sheet (to lay out for floor protection)

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, glove)

Operation Process

# 1. Arrival of lorry loader onsite and park at designated location

# 2. To unload required work tools from lorry

# 3. Bring required work tools to intended location inside factory

# 4. Prepared hydraulic jack to jack up machine

# 5. Insert hydraulic jack under jacking point of machine

# 6. Jack up one side of machine to 3” high and lowered down onto solid woodblock

# 7. Jack up other side of machine and insert rollers under machine bed

# 8. Release jack after rollers are secured onto machine bed

# 9. Shift out machine from existing location

# 10. Use of forklift to assist in pulling of machine to new location

# 11. Position machine at the designated location

# 12. Insert hydraulic jack under jacking point of machine

# 13. Jack up machine to 3” high

# 14. Remove rollers and insert solid woodblock under machine bed

# 15. Release jack and let machine sit onto solid woodblock

# 16. Jack up other side of machine to remove rollers under machine bed

# 17. Insert levelling pad onto machine bed

# 18. Jack up other side of machine

# 19. Remove woodblock and insert levelling pad onto machine bed

# 20. Release and lower down jack slowly and let machine sit onto levelling pad

# 21. Repeat process till all machine are being shift to new designated location

# 22. Once job complete, team leader will instruct the team to perform proper housekeeping

# 23. All debris will be clear and dispose

# 24. Job complete

# Best Regards

# Zailani / Kevin Wong

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175