# METHOD OF STATEMENT

Date: 22nd Sep 2022

Description of work

To relocate machineries from existing location to another location within factory premise

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/09/22 (Phase 1)

1 unit Amada 358 Turret Punch

1 unit Time Saver

14-16/10/22

(Phase 2)

1 unit Aida 250ton Press

1 unit Chin Fong 300ton Press

2 unit Chin Fong 250ton Press

1 unit 150ton Single Crank Press

3 unit 110ton Press

1unit Amada 150ton Double Crank Press

Phase 3

1 unit Komatsu 150ton Single Crank Press

1 unit Aida 110ton Press

2 unit Chin Fong 110ton Press

1 unit Shing Tai 110ton Press

3 unit OCP-80N Press

1 unit 45ton Press

Time of Operation

09:45 am

Location of Operation

#### 36 Joo Koon Circle

Trend Technologies (S) Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing location

# Transport to new location within factory premise

# Position machine at the new designated location

# Unload machine from rollers and insert levelling pad onto machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175