# METHOD OF STATEMENT

Date: 20th June 2025

Description of work

To shift out 4 units machine production to carpark area

Machine Spec

1 unit Chin Feng GTX-400

1 unit Aida PMX-200

1 unit Komatsu E2M-300

1 unit Komatsu E2M-200

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

23/06/25

Time of Operation

09:30 am

Location of Operation

#### 38 Benoi Place

Civic International Pte Ltd

Equipment Use

Hydraulic jack

Woodblock

Motor operated rollers/manual rollers

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe, safety helmet)

Job Scope

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from production

# Transport to temporary location at carpark area

# Position machine at designated location

# Jack up one side of machine

# Remove rollers and insert woodblock under machine bed

# Jack up another side of machine, remove rollers and insert woodblock under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175