# METHOD OF STATEMENT

Date: 17th Sep 2024

Description of work

To relocate 4 unit machine from #05-01 to #05-02

Machine Spec

3 unit Grinder machine

1 unit Lathe machine

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

20/09/24

Time of Operation

10:00 am

Location of Operation

#### Yong Chang Mould Engineering Pte Ltd

Blk 5012 A.M.K Ave 5

#### From #05-01 to #05-02

Equipment Use

Hand jack/crowbar

Woodblock

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Lift up machine to 3” high

# Insert pallet truck under machine bed

# Shift out machine from existing unit

# Transfer machine to new unit

# Position machine at the designated location

# Lift up machine to remove pallet truck and

# Insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175