# METHOD OF STATEMENT

Date: 07th June 2023

Description of work

To relocate 2 unit machine from existing location to another location within factory premise

Machine Spec

1 unit 250ton High Speed Hydraulic Mould Press

Wt: 14, 000 kg

1 unit HM300 High Speed Hydraulic Mould Press

Wt: 14, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

15/06/23

Time of Operation

09:45 am

Location of Operation

#### 10 Tuas South St 5

CDI Energy Products Pte Ltd

Equipment Use

1 x lorry loader

Hydraulic/hand jack

Woodblock

Rollers

1 x battery forklift

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Deploy lorry loader at designated unloading area

# Hoist down forklift from lorry chassis to the ground

# Jack up machine to 4” high

# Insert rollers onto machine bed

# Shift out machine from existing location

# Transport to new location within factory premise

# Use of battery to assist in pulling/pushing machine

# Position machine at the new designated location

# Jack up one side of machine

# Remove rollers and insert hard wooden block under machine bed

# Jack up another side of machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Zailani / Kevin Wong

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175