# METHOD OF STATEMENT

Date: 01st Dec 2025

Description of work

To hoist down 1 unit machine from lorry chassis, load onto platform, shift and position into tenant premise at ground

Machine Spec

1 unit Vertical Storage System

Dim: 2680 x 1400 x 2540 mm

Wt: 1, 650 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/12/25

Time of Operation

Estimate:

11:30 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #01-12

Amax Precision (S) Pte Ltd

Equipment Use

2 x lorry crane

1 x platform - to lay out for machine pathway

Rollers/pallet truck

Wire sling

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

c. Signalman

d. Riggerman

e. Skill Mover

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Unload and lay out platform in front of tenant shutter door opening for machine pathway

- Hoist up machine from lorry chassis and load onto platform

- Insert pallet truck/rollers onto machine bed

- Shift out machine from platform

- Transport machine into tenant premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175