# METHOD OF STATEMENT

Date: 18th May 2018

Description of work

To unload 2 unit machine from lorry chassis onto platform, insert pallet truck/rollers to machine base, shift and position into premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23/05/2018   -   09:30 AM

Location of Operation

#### 21 Kallang Ave #01-171

Amethyst Micrometric (S) Pte Ltd

Equipment Use

2 x lorry crane

1 x platform - to lay out for machine pathway

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated loading area for the lift

- Hoist down and lay out platform for machine pathway

- Hoist down machine from lorry chassis and load onto platform

- Insert pallet truck/rollers under machine base

- Move out machine from platform

- Shift and position machine into designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175