# METHOD OF STATEMENT

Date: 02nd Apr 2018

Description of work

To shift out 1 unit machine with accessary from production room at ground floor, hoist up and load onto 1 x 40ft HQ container

Machine Spec

1 unit LC38/3000 CNC Lathe

Dim: 600 x 210 x 210 cm

Wt: 15, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

06/04/2018   -    09:30 am

Location of Operation

#### Blk 10 Kaki Bukit Ave 1 #01-02

Super Union Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x platform - to lay out for machine pathway

Heavy duty rollers

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc. safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated area for the lift

- Barricade operation area with safety cones, red white tape

- To lay out platform for machine pathway

- Shift out machine from production room

- Load machine into platform

- Hoist up machine and load onto container

- Secured machine with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

To dismantle wooden door frame and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175