# METHOD OF STATEMENT

Date: 17th Feb 2022

Description of work

1) To shift out 2 unit machine from production room into platform, hoist up and load onto forwarder lorry

2) To unload/hoist down 2 unit new machine from lorry chassis, shift and position into tenant premise

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/03/22 – 02/04/22

Time of Operation

09:45 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #01-02 & #01-03

Progress Engineering Services Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x forwarder lorry

1 x platform - to lay out in front of tenant door for machine pathway

Rollers

Pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location for the lift

- Barricade off work area with safety cones, red white tape and lifting operation signage

- Unload and lay out platform in front of tenant shutter door opening for machine pathway

- Shift out old machine from production room into platform

- 1 x lorry crane to hoist up machine from platform

- Load machine onto forwarder lorry and secured with strapping belt prior to moving out of  premise

- To hoist down new machine from lorry chassis onto platform

- Insert rollers under machine bed

- Shift and transport into tenant premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175