# METHOD OF STATEMENT

Date: 10th June 2019

Description of work

To shift out 1 unit Mitsubishi Uv1 CNC Milling machine from production room onto platform, hoist up and load onto 1 x 20ft Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/06/2019

Time Of Operation

09:30 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #01-11/12

Amax Precision (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x platform - to lay out for machine pathway

1 x 20ft Malaysia lorry

Rollers / pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated area for the lift

- Barricade operation area with safety cones, red white tape

- Lay out platform for machine pathway

- Shift out machine from production room into platform

- 1 x lorry crane to hoist up machine from platform

- Load machine onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of  premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175