# METHOD OF STATEMENT

Date: 11th Nov 2024

Description of work

To shift out 6 unit machine and accessories from production room into platform, hoist up and load onto 2 x M’sia lorry and 2 x M’sia low-bed

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/11/24 (Wednesday)

Time of Operation

10:00 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #01-12

Amax Precision (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x platform - to lay out for machine pathway

Rollers / pallet truck

Hand jack/woodblock

1 set wire sling

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated area for the lifting work

- Barricade operation area with safety cones, red white tape and lifting in operation signage

- Hoist down platform from lorry chassis

- Lay out platform in front of tenant shutter gate opening for machine pathway

- Shift out machine from production room

- Load machine into platform

- Riggers to secured sling to crane hook

- Secured sling to rigging point of machine

- 1 x lorry crane to hoist up machine from platform

- Load machine onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of  premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175