# METHOD OF STATEMENT

Date: 01st Apr 2020

Description of work

To hoist down 1 unit CNC Lathe machine from lorry chassis, load onto platform, shift and position into premise at ground

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

04/04/2020 (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 3015A Ubi Rd 1 #01-11

Hao Yang Technologies Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x 30 ton lorry crane

1 x platform - to lay out directly in front of tenant door for machine pathway

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- Lay out platform directly in front of tenants door for machine pathway

- Hoist down machine from lorry chassis and load onto platform

- Insert rollers under machine base

- Shift out machine from platform

- Position machine into tenant premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175