# METHOD OF STATEMENT

Date: 25 September 2015

Description of work

To shift out machineries from 2nd & 5th floor via cargo lift, hoist up and load onto lorry chassis.

17 unit Bar Feeder to be hoist down by using platform and mobile/lorry crane.

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

03/10/2015 - 09:30 am   (14 unit machine via cargo lift and 10 unit Bar Feeder by hoisting)

10/10/2015 - 09:30 am   (15 unit machine via cargo lift and 7 unit Bar Feeder by hoisting)

Location of Operation

#### Blk 9 Kallang Place #02-08 & #05-01

Klas Industries Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

1 x platform

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated area for the lift

- Barricade operation area with safety cones, red white tape

- Hoist up platform to 5th floor opening

- Stabilize platform and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Push bar feeder into platform

- 50 ton mobile crane to hoist down platform together with bar feeder and load onto ground

- Lorry crane to hoist up bar feeder from platform and load onto lorry chassis

- Machine to be properly secured prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175