# METHOD OF STATEMENT

Date: 14th Aug 2024

Description of work

To shift out 1 unit machine and accessories from production area to loading bay, hoist and load onto customer arrange container

Machine Spec

VTL 1250 machine (dismantle to 5 parts)

1 unit bed / Wt: 10, 000 kg

1 unit column / Wt: 12, 000 kg

1 unit atc housing / Wt: 5, 000 kg

1 unit cross beam / Wt: 4, 400 kg

1 unit chip conveyor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

2 day job

Time of Operation

09:45 am

Location of Operation

#### 2 Benoi Rd

Vetco Gray Pte Ltd

Equipment Use

1 x 75ton lorry loader

633 lorry loader

1 x 3 ton battery forklift

Hydraulic/hand jack

Electrical rollers/rollers

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting location

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Using hydraulic jack to jack up machine to 4” high

- Insert rollers under machine bed

- To shift out machine from production to loading area

- Riggers to attached sling to crane hook

- Secure sling to rigging point of machine

- To hoist up machine 1ft above ground to check on stability

- When well balance, hoist and load onto container

- Release all sling from rigging point of machine

- Secured machine with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175