# METHOD OF STATEMENT

Date: 14th Aug 2024

Description of work

To shift out 1 unit machine and accessories from production area to loading bay, hoist up onto wooden base for packing, after packing, load onto 1 x 40ft FR and 1 x 40ft HQ container

Machine Spec

Mazak Slant Turn Nexus 550M

Dim: 730 x 275 x 250 cm

Wt: 19, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

09:45 am

Location of Operation

#### 2 Benoi Rd

Vetco Gray Pte Ltd

Equipment Use

1 x 50ton mobile crane

633 lorry loader

1 x boom lift

1 x low-bed

1 x 3 ton battery forklift

Hydraulic/hand jack

Electrical rollers/rollers

Wire/webbing sling

Shackle

Metal plate/canvas (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile crane at designated hoisting location

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Using hydraulic jack to jack up machine to 4” high

- Insert rollers under machine bed

- To shift out machine from production to loading area

- Riggers to attached sling to crane hook

- Secure sling to rigging point of machine

- To hoist up machine 1ft above ground to check on stability

- Load up machine onto wooden for packing

- Pack to form a case

- After packing hoist and load up case onto container

- Secured case with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175