# METHOD OF STATEMENT

Date: 17th Feb 2023

Description of work

To shift out 2 unit machine and accessories from production area to loading bay, hoist up onto wooden base for packing, after packing, load onto 2 x 40ft FR and 1 x 40ft HQ container

Machine Spec

Mazak FH6800 CNC

Dim: 575 x 300 x 280 cm

Wt: 19, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01-03/03/23

Time of Operation

09:30 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 100 ton lorry loader

633 lorry loader

1 x low loader

1 x 3 ton battery forklift

Hydraulic/hand jack

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Shift out machine from production to loading area

- Attached lifting gear to crane hook and secure to rigging point of machine

- Hoist up machine and load onto wooden

- Pack to form a case

- After packing hoist up case onto low loader

- Shift out case to outside factory

- Hoist up and transfer case from low loader onto container

- Secured case with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175