# METHOD OF STATEMENT

Date: 20th Jan 2025

Description of work

To shift out machine and accessories from production area to outside, hoist up onto wooden base for packing

Spec

Refer to quotation

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

As per required schedule

Time of Operation

09:45 am

Location of Operation

John Crane

15 Tuas view place

Equipment Use

633 lorry loader

Pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting location

- To shift out hachuic from production to loading area

- To attached sling to crane hook

- Secure sling to rigging point of manipulator

- Hoist up machine

- Load onto wooden for packing

- Pack to form a case

- After packing hoist and load up case onto container and secured

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175