# METHOD OF STATEMENT

Date: 27th July 2018

Description of work

To shift out 1 unit machine from tool room to loading area, hoist up onto lorry chassis and deliver to new building, unload, shift and position into premise at ground floor

Machine Spec

1 unit V850G

Dim: 2840 x 2900 x 2290 mm

Wt: 4, 600 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

01/08/2018   –   09:45 am

Location of Operation

#### 50 Nanyang Ave, HW 3

Nanyang Technological University

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine using crowbar

- Insert rollers/pallet truck under machine bed

- Shift out machine from tool room to loading bay

- Hoist up and load machine onto lorry chassis

- Secured machine with belt prior to moving out of premise and deliver to new building

- At new location, hoist down machine from lorry chassis, insert rollers/pallet truck, shift and position into premise at ground floor

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping,

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175