# METHOD OF STATEMENT

Date: 28th June 2017

Description of work

To shift out 2 unit Furnace from tool room to loading area, hoist up onto lorry chassis and deliver to new location

Furnace Dimension

80 x 70 x 170 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

03/07/2017   –   09:45 am

Location of Operation

#### 50 Nanyang Ave, Blk N4.1  -  Blk N3.1

Nanyang Technological University

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, if any will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up furnace using crowbar

- Insert pallet truck

- Shift out furnace from tool room to loading bay

- Hoist up and load furnace onto lorry chassis

- Secured furnace with belt prior to moving out of premise and deliver to new location

- At new location, unload furnace from lorry chassis, shift and position into premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping,

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175