# METHOD OF STATEMENT

Date: 15th Feb 2017

Description of work

To shift out 1 unit set of American Isostatic machine from tool room to loading area, hoist up onto lorry chassis and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 01-04/02/2017   –   10:00 am

Location of Operation

#### From Blk HW1, 2A Nanyang Link   –   New Academic Block North

Equipment Use

1-2 x lorry crane

Low loader

Pallet truck

Webbing sling

A Frame with chain block

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, if any will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high using crowbar

- Insert pallet truck to machine base

- Shift out machine from tool room to loading bay

- Assemble ‘A’ frame with chain block to assist in dismantling machine hoist structure

- Hoist up and load machine onto lorry chassis

- Secured machine with belt prior to moving out of premise and deliver to new location

- At new location, unload machine from lorry chassis, shift and position into premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping,

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175