# METHOD OF STATEMENT

Date: 22nd Feb 2021

Description of work

To shift out 1 unit machine from workshop area to loading bay, hoist up onto lorry chassis and deliver to another location

Machine Spec

1 unit Makino Lathe machine

Dim: 165 x 100 x 140 cm

Wt: 1, 350 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/02/2021

Time of Operation

Afternoon schedule

Location of Operation

#### 1 Chua Chu Kang Grove

ITE College West, Blk 5, School of Engineering

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Shift out machine from workshop to loading area

- Attached wire sling to crane hook and secure to machine hoisting point

- Hoist up machine about 1ft above ground to check on balancing

- When well balance, hoist up machine and load onto lorry chassis

- Release all sling from machine

- Secured machine with strapping belt prior to moving out of premise and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175