# METHOD OF STATEMENT

Date: 17th June 2021

Description of work

To shift out 1 unit machine from ground floor to loading area, hoist up onto lorry chassis and deliver to new factory

Machine Spec

1 unit 7.10.7

Dim: 1277 x 1908 x 2777 mm

Wt: 1, 265 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

09:45 am

Location of Operation

#### 18 Teban Gardens Crescent

Setsco Services Pte Ltd

Equipment Use

1 x lorry crane

1 x battery lifter

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Insert pallet truck to machine base

- Shift out machine from CMM room to loading bay

- Insert webbing sling onto machine hoisting point

- Lorry loader to hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of old premise

- Transport and deliver machine to new location

- At new factory, unload machine, shift and position into factory premise at ground floor

- Using battery lifter to lift up granite table for your engineer to insert pad and lower back onto stand

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175