# METHOD OF STATEMENT

Date: 10th Sep 2021

Description of work

To shift out 1 unit machine to loading area, hoist up onto lorry chassis and deliver to main building

Machine Spec

1 unit 6.8.6 CMM

Dim: 1150 x 1623 x 2638 mm

Wt: 730 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/09/21 (Monday)

Time of Operation

09:45 am

Location of Operation

#### 501 AMK Industrial Pk 1

Borg Warner Singapore Holding Pte Ltd

Equipment Use

1 x lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Insert pallet truck to machine base

- Shift out machine to loading area

- Insert webbing sling onto machine hoisting point

- Lorry loader to hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver machine to main building

- Reverse lorry to loading bay

- Shift out machine from lorry chassis

- Transport machine to upper floor building via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175