# METHOD OF STATEMENT

Date: 28th Aug 2020

Description of work

To shift out 1 unit CMM machine from production area to loading bay, hoist up and load onto lorry chassis, transport to Eric Wong warehouse for scrap purpose

Machine Spec

1 unit Zeiss MMZ 16.30.16 CMM

Dim: 460 x 260 x 210 cm

Wt: 10, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/09/20-08/09/20

Time of Operation

9.30am

Location of Operation

#### 51 Corporation Rd

Avitron Pte Ltd

Equipment Use

2 x 633 PALFINGER lorry loader

Rollers/pallet truck

Webbing sling

Woodblock

Hydraulic/hand jack

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine using hand/hydraulic jack

- Insert rollers onto machine base

- Shift out machine from production to loading area

- Attached required lifting gears onto crane hook and secured to rigging point of machine to be lifted

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175