# METHOD OF STATEMENT

Date: 02nd June 2020

Description of work

To shift out 1 unit CMM machine from production area to loading bay, hoist up and load onto Malaysia low-bed

Machine Spec

1 unit Gemini CMM

Dim: 350 x 270 x 350 cm

Wt: 10, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

09/06/20

Time of Operation

09:30 am

Location of Operation

#### 61 Seletar Aerospace

Pratt & Whitney

Equipment Use

1 x 633 lorry loader

Rollers/Air rollers

Webbing sling

Woodblock

Hydraulic jack

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine using hand/hydraulic jack

- Move out machine stand

- Insert rollers onto machine base

- Shift out machine from production to loading area

- Hoist up and load machine onto Malaysia low-bed

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175