# METHOD OF STATEMENT

Date: 22nd Sep 2022

Description of work

1) To shift out 1 unit machine from production to loading area, hoist up onto lorry chassis and deliver to another location

2) To relocate 1 unit machine for clearance and move back after machine move out

Machine Spec

1 unit Lapping machine

Dim: 170 x 208 x 152 cm

Wt: 2, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/09/22

Time of Operation

09:45 am

Location of Operation

#### 8 Tractor Rd

Excelitas Technologies (S) Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Insert pallet truck under machine base

- Shift out machine from production room to loading bay

- Insert sling to crane hook and secure onto machine hoisting point

- Lorry loader to hoist up machine

- Load machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175