# METHOD OF STATEMENT

Date: 23rd Feb 2023

Description of work

To shift out 1 unit machine and accessories from production area to loading bay, hoist up onto low loader and transport to another location

Machine Spec

Youji YV1000ATC Vertical Turning Lathe

Dim: 380 x 380 x 410 cm

Wt: 19, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/03/23

Time of Operation

09:40 am

Location of Operation

#### 2A Changi North St 2

ETLA Limited

Equipment Use

1 x 50 ton mobile crane

633 lorry loader

1 x low loader

1 x 3 ton battery forklift

Hydraulic/hand jack

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated loading area

- Shift out machine from production to loading area

- Attached lifting gear to crane hook and secure to rigging point of machine

- Hoist up machine

- Unload machine onto low loader chassis

- Release all sling from rigging point of machine

- Secure machine with strapping belt

- Transport machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175