# METHOD OF STATEMENT

Date: 01st Apr 2019

Description of work

To shift out 16 unit machine and accessories from tenant premise to loading bay, hoist up and load onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

02/04/2019

Time Of Operation

09:40 am

Location of Operation

#### 31 Senoko South Rd

Santak Industrial

Equipment Use

1-2 x lorry crane

Pallet truck/rollers

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine using crowbar

- Insert pallet truck to machine base

- Shift out machines from tool room to loading area

- Hoist up machine and load onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175