# METHOD OF STATEMENT

Date: 15th May 2023

Description of work

To shift out 1 unit machine from production to loading area for packing using existing base, after pack load up onto 1 x M’sia lorry

Machine Spec

Bruderer BSTA 30llH

Dim: 190 x 110 x 180 cm

Wt: 5, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/05/23

Time of Operation

09:30 am

Location of Operation

#### 65 Loyang Way

#### Bruderer Presses (Far East) Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x platform (to lay out for machine pathway)

Rollers/pallet truck

Wire/webbing sling

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out machine from production to loading area

- Attached required sling to crane hook and secured to rigging point of machine

- Hoist up machine and load onto existing base

- After pack, load up onto 1 x M’sia lorry

- Secured with strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175