# METHOD OF STATEMENT

Date: 11th Nov 2022

Description of work

To shift out 2 unit machine from production area to loading bay, hoist up onto wooden base for packing, after packing place aside

Machine Spec

1 unit Mazak Power Master

Dim: 590 x 250 x 225 cm

Wt: 11, 000 kg

1 unit Mazak Slant Turn 50N

Dim: 560 x 266 x 265 cm

Wt: 14, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14-15/11/22

Time of Operation

09:45 am

Location of Operation

#### 149 Gul Circle

#### FMC Technologies Singapore Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling/shackle

Packing materials

Hand grinder (to cut low plug after machine move out)

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Shift out machine from workshop to loading area

- Riggers to attached all required lifting gears to crane hook and secure onto rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- Load up machine onto wooden base

- Pack to form a case

- After packing, place case aside

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175