# METHOD OF STATEMENT

Date: 22nd July 2019

Description of work

To shift out 2 unit machine and accessories from production area to loading area, fork up machine onto wooden base for packing, after packing shift and put aside

Machine Spec

2 unit Gisholt Chopping machine

Main Dim: 420 x 195 x 260 cm / Wt: 7, 000 kgs

Oil tank: 130 x 80 x 175 cm

Chip tray: 115 x 120 x 90 cm

Main Dim: 420 x 195 x 195 cm / Wt: 7, 000 kgs

Oil tank: 300 x 55 x 175 cm

Chip tray: 115 x 120 x 90 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/07/2019   -    To shift out machine parts and accessories for packing

26/07/2019   -    To shift out main machine from production to loading bay for packing

Time of Operation

09:45 am

Location of Operation

No 4 Gul Way

National Oilwell Varco Pte Ltd

Equipment Use

1-2 x lorry crane

Rollers/pallet truck

Hand jack

1 set wire sling

1 x 10 ton forklift

Obstacles

No obstacle on crane parking and packing area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine using jack

- Insert rollers/pallet truck under machine bed

- Shift out from production to loading area

- Forklift to fork up machine and load onto wooden base

- Pack machine into crate

- After packing, fork case and put aside

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175