# METHOD OF STATEMENT

Date: 01st Aug 2025

Description of work

To shift out 1 unit machine from production to loading area, using forklift to fork up onto lorry chassis and deliver to new location

Machine Spec

1 unit Drill machine

Dim: 130 x 100 x 270 cm

Wt: 1, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/08/25

Time of Operation

09:40 am

Location of Operation

#### 249 Jalan Boon Lay

ST Engineering

Equipment Use

1 x lorry crane

1 x 2.5ton forklift

Pallet truck

Crowbar/woodblock

Wire/webbing sling

1 x hand grinder

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Hoist down forklift from lorry chassis

- Using forklift to assist engineer dismantling parts from machine

- Shift out machine from production room to loading area

- Using forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175