# METHOD OF STATEMENT

Date: 23rd May 2025

Description of work

To shift out 1 unit machine from production to loading area, using forklift to fork up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

1 unit Ultrasonic machine

Dim: 500 x 175 x 220 cm

Wt: 3, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

31/05/25

Time of Operation

09:40 am

Location of Operation

#### 8 Tractor Rd

Excelitas Technologies (S) Pte Ltd

Equipment Use

1 x lorry crane

1 x 4.5ton forklift

Rollers/pallet truck

Crowbar/woodblock

Wire sling

Plywood/PVC plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Hoist down forklift from lorry chassis to the ground

- To lift up machine and insert woodblock under machine bed

- Insert pallet truck under machine bed

- Shift out machine from production room to loading area

- Using forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175