# METHOD OF STATEMENT

Date: 11th June 2025

Description of work

To shift out 1 unit machine and accessories from basement to 2nd floor loading bay via cargo lift, load onto lorry chassis and deliver to new location, unload using forklift, shift and position into premise at ground floor

Machine Spec

1 x Hammer C3 41 Combination Saw / Wt: 800 kg

1 x table

1 x dust collector

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/06/25

Time of Operation

09:30 am

Location of Operation

#### NUS

Move out: 18 College Ave West

Move in: 2 Engineering Dr 3, Blk E3 #B1-06

College Of Design & Engineering, Central Workshop

Equipment Use

1 x lorry crane

1 x 3ton forklift

Pallet truck

Crowbar/woodblock

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator will be involved in this operation

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- To lift up machine and insert woodblock under machine bed

- Insert pallet truck under machine bed

- Shift out machine from basement to loading bay via cargo lift

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver to new location

- Using forklift to fork down machine from lorry chassis

- Unload machine to the ground

- Using pallet truck to shift machine into premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175