# METHOD OF STATEMENT

Date: 19th Feb 2025

Description of work

To shift out following machine from production at ground floor to loading area, hoist and load up onto buyer lorry

Machine Spec

Refer to attached machine list

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 8 Tractor Rd

Excelitas Technologies (S) Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Crowbar

Woodblock

Wire/webbing sling

PVC sheet/plywood (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading area

- Lift up machine and insert woodblock under machine bed

- Insert pallet truck under machine bed

- Shift out machine from production room

- Transport to loading bay

- Rigger to insert sling to crane hook

- Secure sling to machine rigging point

- Lorry crane to hoist up machine

- Load up machine onto buyer lorry

- Release all sling from crane hook

- Secured machine with strapping belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175