# METHOD OF STATEMENT

Date: 23rd Aug 2024

Description of work

1) To shift out 1 Schaubt CG machine from production to loading area, hoist up onto lorry chassis and deliver to scrapyard for disposal

2) To relocate 1 unit Faisei Grinder within workshop

3) To shift out 1 unit C clamp machine from existing building, hoist up onto lorry chassis, transfer to another building, unload, shift and position workshop at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

02/09/24

Time of Operation

10:00 am

Location of Operation

#### 501 Airport Rd

ST Aerospace Engines Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Crowbar/hand jack

Woodblock

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in the operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

(1)

Deploy lorry loader at designated loading area

Set up crane with outriggers fully extended and resting on steel plate/timber mats as base

plate

Jack up machine and let it sit onto woodblock

Insert pallet truck under machine bed

To shift out machine from production to loading area

Rigger to insert sling to crane hook

Secured sling to rigging point of machine

Once rigging is done, signalman to give signal to crane operator to hoist up machine

Load up machine onto lorry chassis

Secured machine with strapping belt

Transport and deliver to scrapyard for disposal

(2)

Lift up machine using crowbar and let it sit onto woodblock

Insert pallet truck under machine bed

Shift out from existing location

Transfer to new location within workshop

Position at the designated location

(3)

Lift up machine using crow and let it sit onto woodblock

Insert pallet truck under machine bed

To shift out machine from production to loading area

Rigger to insert sling to crane hook

Secured sling to rigging point of machine

Once rigging is done, signalman to give signal to crane operator to hoist up machine

Load up machine onto lorry chassis

Secured machine with strapping belt

Transport to another building within premise

Unload, shift and position into premise at ground floor

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175