# METHOD OF STATEMENT

Date: 13th May 2024

Description of work

To shift out 2 unit machine from production area to loading bay, hoist up onto lorry chassis and deliver to another location

Machine Spec

1 unit Lathe machine

Dim: 530 x 220 x 195 cm

Wt: 9, 000 kg

1 unit Lathe machine

Dim: 630 x 325 x 220 cm

Wt: 15, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/05/24

Time of Operation

09:45 am

Location of Operation

#### 149 Gul Circle

#### FMC Technologies Singapore Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x low-bed

Hydraulic/hand jack

Woodblock

Rollers

Wire/webbing sling

Shackle

Hand grinder (to cut stud)

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Jack up machine to 4” high and let it sit on solid woodblock

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Riggers to attached required sling to crane hook

- Secure sling onto rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- Load machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175