# METHOD OF STATEMENT

Date: 14th Oct 2024

Description of work

To shift out 1 unit machine from production area to loading bay, hoist up onto lorry chassis and deliver to another location

Machine Spec

1 unit Power Master

Dim: 560 x 250 x 225 cm

Wt: 11, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

05/11/24

Time of Operation

09:45 am

Location of Operation

#### 149 Gul Circle

#### FMC Technologies Singapore Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x forklift

Hydraulic jack

Woodblock

Rollers/electrical roller

Wire sling

Shackle

Hand grinder (to cut off stud)

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Jack up machine to 4” high and let it sit on solid woodblock

- Cut off all stud

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Riggers to attached required sling to crane hook

- Secure sling onto rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- Load machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt

- Transport and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175